In-Class Stress-Strain Activity – Materials and Instructor Notes

Slotted Weight Set from Pasco - https://www.pasco.com/products/lab-supplies/mass-sets/large-slotted-mass-set

Obviously there can be plenty of options to find some way to hanging variable weights (things in the 0.5-2.5 kg range in increments of 0.5 kg work well)

Rubber Rods - https://www.mcmaster.com/products/rubber-rods/oil-resistant-buna-n-rubber-rods/

- I personally used ¼”, 3/8”, and ½” 50A and 60A durometer rods and it worked well. They come in 36” lengths, and I cut them down to either 6” or 9”.

- The first time I ran this, I used 4 different geometries of all the same material (thin, medium, medium short, thick). The second time, I only used three sizes (thin, medium, thick all of the same length) and the medium size was made from the stiffer 60A durometer material. You could theoretically do it with 4 rods AND different materials, depending on the level of complexity you wanted.

Vinyl-Coated Ceiling Hooks - https://www.amazon.com/Ceiling-ULIFESTAR-Coated-Storage-Organizer/dp/B07VPLZXJ9?th=1

Can obviously find these everywhere, but I found screwing them into the bottom of the rods is an excellent way to hang the weights

Rulers, Measuring Tapes, and/or Calipers

For having students measure diameter and length.